# Fieldbook Report

10/06/2022 18:25:17

## Job Information

| Job name: | MATEBELE2022 |
| --- | --- |
| Created: | 10/06/2022 07:54:48 |
| Description: | CAD |
| Creator: | BISM |
| Time zone: | 2h 00' |
| Coordinate system name: | MATEBELE2D |
| Application software: | LEICA Geo Office 7.0 |
| Firmware version: | 5.60 |
| Codelist name: | THEBE |
| Average limit (Position): | 0.0500 m |
| Average limit (Height): | 0.0750 m |

## Coordinate System Information

| Coordinate system name: | MATEBELE2D |
| --- | --- |
| Created: | 08/24/2018 11:37:24 |
| Transformation name: | MATEBELE2D |
| Transformation type: | Twostep |
| Height mode: | Orthometric |
| Pre-transformation name: | DSM_BNGR_To_BTRS |
| Residuals: | 1 / ( distance^2 ) |
| Local Ellipsoid: | Clarke 1880 |
| Projection: | TM27 |
| Geoid model: | - |
| CSCS model: | - |
| ## Transformation details 2D-Helmert transformation. Number of common points:; 4. Rotation origin:; X0:; -0.0041 m. Y0:; -0.0047 m. No.; Parameter; Value. 1; dE; 2370216.5416 m. 2; dN; -47706.6053 m. 3; Rotation; -179° 49' 43.18702". 4; Scale; -4.1346 ppm. Height transformation. Number of common points:; 0. Mean transformation accuracy:; 0.0000 m. Parameters:; 0.00000000; 0.00000000; 0.0000 m. Inclination of height in X:; 0° 00' 00.00000". Inclination of height in Y:; 0° 00' 00.00000". ## Residuals Grid:. System A; System B; Point type; dE [m]; dN [m]; dHgt [m]. MTRM1; MTRM1; Position; 0.0177 m; -0.0054 m; -. MTRM4; MTRM4; Position; -0.0102 m; -0.0152 m; -. MTRM13; MTRM13; Position; -0.0012 m; 0.0105 m; -. MTRM14; MTRM14; Position; -0.0153 m; 0.0200 m; -. ## List of identical points System A:. WGS 84 Cartesian:. X [m]; Y [m]; Z [m]. MTRM1; 5172619.7024; 2539930.5454; -2216000.7065. MTRM4; 5126915.7150; 2539155.6622; -2215356.3896. MTRM13; 5126593.4674; 2539523.3685; -2215644.4387. MTRM14; 5126282.2375; 2539314.8986; -2216611.0431. System B:. Local Grid:. Easting [m]; Northing [m]; Hgt [m]. MTRM1; 96998.0100; 2714789.3690; -. MTRM4; 97672.8530; 2715614.3960; -. MTRM13; 97047.1310; 2713980.6310; -. MTRM14; 97476.5210; 2713648.9690; -. |  |

## GPS Coordinates

| Baseline |  | Reference: MTRM4 | Rover: 25A1 |
| --- | --- | --- | --- |
| Local Coordinates: |  |  |  |
|  | Easting: | 97672.8530 m | 97039.6447 m |
|  | Northing: | 2715614.3960 m | 2715189.9822 m |
|  | Ellip. Hgt: | - | - |

| Quality: | Sd. E: 0.0081 m | Sd. N: 0.0066 m | Sd. Hgt: 0.0180 m |
| --- | --- | --- | --- |
|  | Posn. Qlty: 0.0104 m | Sd. Slope: 0.0082 m |  |

| Baseline |  | Reference: MTRM4 | Rover: 25A2 |
| --- | --- | --- | --- |
| Local Coordinates: |  |  |  |
|  | Easting: | 97672.8530 m | 96994.6643 m |
|  | Northing: | 2715614.3960 m | 2715195.3046 m |
|  | Ellip. Hgt: | - | - |

| Quality: | Sd. E: 0.0178 m | Sd. N: 0.0300 m | Sd. Hgt: 0.0323 m |
| --- | --- | --- | --- |
|  | Posn. Qlty: 0.0349 m | Sd. Slope: 0.0152 m |  |

| Baseline |  | Reference: MTRM4 | Rover: 25A3 |
| --- | --- | --- | --- |
| Local Coordinates: |  |  |  |
|  | Easting: | 97672.8530 m | 96987.8468 m |
|  | Northing: | 2715614.3960 m | 2715157.2603 m |
|  | Ellip. Hgt: | - | - |

| Quality: | Sd. E: 0.0042 m | Sd. N: 0.0034 m | Sd. Hgt: 0.0093 m |
| --- | --- | --- | --- |
|  | Posn. Qlty: 0.0055 m | Sd. Slope: 0.0043 m |  |

| Baseline |  | Reference: MTRM4 | Rover: 25A4 |
| --- | --- | --- | --- |
| Local Coordinates: |  |  |  |
|  | Easting: | 97672.8530 m | 97027.2856 m |
|  | Northing: | 2715614.3960 m | 2715150.1988 m |
|  | Ellip. Hgt: | - | - |

| Quality: | Sd. E: 0.0126 m | Sd. N: 0.0074 m | Sd. Hgt: 0.0354 m |
| --- | --- | --- | --- |
|  | Posn. Qlty: 0.0147 m | Sd. Slope: 0.0124 m |  |

| Baseline |  | Reference: MTRM4 | Rover: M1 |
| --- | --- | --- | --- |
| Local Coordinates: |  |  |  |
|  | Easting: | 97672.8530 m | 96991.1062 m |
|  | Northing: | 2715614.3960 m | 2715175.4782 m |
|  | Ellip. Hgt: | - | - |

| Quality: | Sd. E: 0.0143 m | Sd. N: 0.0082 m | Sd. Hgt: 0.0379 m |
| --- | --- | --- | --- |
|  | Posn. Qlty: 0.0165 m | Sd. Slope: 0.0084 m |  |

| Baseline |  | Reference: MTRM4 | Rover: M2 |
| --- | --- | --- | --- |
| Local Coordinates: |  |  |  |
|  | Easting: | 97672.8530 m | 97033.6750 m |
|  | Northing: | 2715614.3960 m | 2715170.8772 m |
|  | Ellip. Hgt: | - | - |
| Quality: | Sd. E: 0.0050 m | Sd. N: 0.0042 m | Sd. Hgt: 0.0108 m |
|  | Posn. Qlty: 0.0065 m | Sd. Slope: 0.0049 m |  |

| Baseline |  | Reference: MTRM4 | Rover: WP1 |
| --- | --- | --- | --- |
| Local Coordinates: |  |  |  |
|  | Easting: | 97672.8530 m | 99368.9833 m |
|  | Northing: | 2715614.3960 m | 2713076.1507 m |
|  | Ellip. Hgt: | - | - |

| Quality: | Sd. E: 0.0059 m | Sd. N: 0.0047 m | Sd. Hgt: 0.0196 m |
| --- | --- | --- | --- |
|  | Posn. Qlty: 0.0075 m | Sd. Slope: 0.0049 m |  |

| Baseline |  | Reference: WP1 | Rover: M1 |
| --- | --- | --- | --- |
| Local Coordinates: |  |  |  |
|  | Easting: | 99368.9845 m | 96991.1192 m |
|  | Northing: | 2713076.1495 m | 2715175.5030 m |
|  | Ellip. Hgt: | - | - |

| Quality: | Sd. E: 0.0076 m | Sd. N: 0.0059 m | Sd. Hgt: 0.0275 m |
| --- | --- | --- | --- |
|  | Posn. Qlty: 0.0096 m | Sd. Slope: 0.0061 m |  |

| Baseline |  | Reference: WP1 | Rover: M2 |
| --- | --- | --- | --- |
| Local Coordinates: |  |  |  |
|  | Easting: | 99368.9845 m | 97033.7094 m |
|  | Northing: | 2713076.1495 m | 2715170.8948 m |
|  | Ellip. Hgt: | - | - |

| Quality: | Sd. E: 0.0143 m | Sd. N: 0.0082 m | Sd. Hgt: 0.0379 m |
| --- | --- | --- | --- |
|  | Posn. Qlty: 0.0165 m | Sd. Slope: 0.0084 m |  |

| Baseline |  | Reference: WP1 | Rover: MTRM13CHK |
| --- | --- | --- | --- |
| Local Coordinates: |  |  |  |
|  | Easting: | 99368.9845 m | 97047.1322 m |
|  | Northing: | 2713076.1495 m | 2713980.6301 m |
|  | Ellip. Hgt: | - | - |

| Quality: | Sd. E: 0.0045 m | Sd. N: 0.0044 m | Sd. Hgt: 0.0126 m |
| --- | --- | --- | --- |
|  | Posn. Qlty: 0.0063 m | Sd. Slope: 0.0044 m |  |

## Mean Coordinates and Differences

Point M1

| Avg. Local Coordinates |  |
| --- | --- |
| Easting: | 96991.1127 m |
| Northing: | 2715175.4906 m |
| Ortho. Hgt: | - |
| CQ: | 0.0017 m |

| Use | Limit exceeded | Reference | Date/Time | Posn. diff [m] | Hgt. diff [m] | Posn. + Hgt. diff [m] |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | MTRM4 | 10/06/2022 08:51:39 | 0.0009 | -0.0011 | 0.0014 |
|  |  | WP1 | 10/06/2022 10:24:45 | 0.0014 | 0.0016 | 0.0021 |
| Point M2 |  |  |  |  |  |  |

| Avg. Local Coordinates |  |
| --- | --- |
| Easting: | 97033.6922 m |
| Northing: | 2715170.8860 m |
| Ortho. Hgt: | - |
| CQ: | 0.0026 m |

| Use | Limit exceeded | Reference | Date/Time | Posn. diff [m] | Hgt. diff [m] | Posn. + Hgt. diff [m] |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | MTRM4 | 10/06/2022 08:56:41 | 0.0024 | 0.0016 | 0.0029 |
|  |  | WP1 | 10/06/2022 10:30:47 | 0.0020 | -0.0013 | 0.0024 |